Corporate Communications Regina Wiechens-Schwake Phone: +49 2772 505-2527 E-mail: wiechens-schwake.r@rittal.de Hans-Robert Koch Phone: +49 2772 505-2693 E-mail: koch.hr@rittal.de Patricia Späth Phone: +49 2772 505-2346 E-mail: spaeht.p@rittal.de Rittal GmbH & Co. KG Auf dem Stützelberg 35745 Herborn, Germany www.rittal.com

Rittal at the Hanover industrial trade fair                 (Hannover Messe)

25 to 29 April 2016

Hall 11, Stand E06

New air/water heat exchangers from Rittal

Hygienic climate control

Production plants in the food and pharmaceutical industries must be designed for hygiene, and this principle also applies to the enclosure technology used.  With its new “Hygienic Design” air/water heat exchangers, Rittal is offering a standardised solution for efficient enclosure climate control in applications where hygiene is critical. Thanks to their IP69K protection category, the units are suitable for use with high-pressure cleaning systems, meeting exacting requirements.

Herborn/Hanover, 25 April 2016 – Hygiene is essential in the food and pharmaceuticals industries. Systems and components that are used in such hygiene-critical areas have to meet the special requirements of hygienic design. That is why Rittal has introduced a new air/water heat exchanger for enclosure climate control.

The design of the new air/water heat exchanger, developed as a wall-mounted unit, meets the fundamentals of hygienic design. For example, a roof pitch of 30° makes it impossible for objects to be placed on top of the enclosure. In addition, liquids run off immediately due to the slope. The external gasket is made of silicone and meets the FDA 21 CFR 177.2600 Guideline. It prevents contamination arising between the housing of the air/water heat exchanger and the enclosure.

The climate solution features a housing made of stainless steel (Euronorm 1.4301) that is brushed on the outside (grain size 400) and with a surface roughness < 0.8 µm. The surface can thus be very easily cleaned. The housing is of IP69K (DIN 40050-9) protection class stands up to decontamination with high-pressure cleaners. Daily cleaning with a steam cleaner is thus not a problem. The air/water heat exchangers can be installed as wall-mounted units on the side or rear panel of an enclosure. Two versions are available, with cooling outputs of 700 and 1,300 watts.

(1,913 characters)



Caption

fri160402100.jpg: With its new “Hygienic Design” air/water heat exchangers, Rittal is offering a standardised solution for efficient enclosure climate control in applications where hygiene is critical.

May be reproduced free of charge. Please name Rittal GmbH & Co. KG as the source.

About Rittal

Rittal, headquartered in Herborn, Germany, is a leading global provider of solutions for industrial enclosures, power distribution, climate control and IT infrastructure, as well as software and services. Systems made by Rittal are deployed across a variety of industrial and IT applications, including vertical sectors such as the transport industry, power generation, mechanical and plant engineering, IT and telecommunications. Rittal is active worldwide with 10,000 employees and 58 subsidiaries.

Its broad product range includes infrastructure solutions for modular and energy-efficient data centres with innovative concepts for the security of physical data and systems. Leading software providers Eplan and Cideon complement the value chain, providing interdisciplinary engineering solutions, while Rittal Automation Systems offer automation systems for switchgear construction.

Founded in Herborn in 1961 and still run by its owner, Rittal is the largest company in the Friedhelm Loh Group. The Friedhelm Loh Group operates worldwide with 18 production sites and 78 international subsidiaries. The entire group employs more than 11,500 people and generated revenues of around €2.2 billion in 2015. In 2016, it was named one of Germany’s leading employers by the Top Employers Institute, for the eighth year running.

For more information, visit www.rittal.com and www.friedhelm-loh-group.com.